Supplementary Materials

Motif analysis identifies structural configurations and observed failure patterns associated with cascade development, providing mechanism-oriented insights beyond terminal severity assessments. Engineering motifs are defined as compact, rule-based representations of infrastructure characteristics, functional dependencies, and failure events that capture hypothesized mechanisms of structural susceptibility and failure propagation. Their identification criteria are derived from the network topology, physical constraints, operational interdependencies, and protection mechanisms represented in the underlying simulation model.

Structural motifs are identified from pre-existing network attributes, whereas propagation motifs are extracted from recorded cascade trajectories. To avoid outcome-dependent identification, terminal severity labels are excluded from motif matching. Motif occurrences are systematically evaluated across 7,800 simulated cascade trajectories in the synthetic Texas power–gas system. Because motif definitions are system-specific, their transferability requires reassessment against the characteristics and operating conditions of other infrastructure systems. The resulting associations provide evidence for interpreting recurrent cascade mechanisms and formulating targeted intervention hypotheses, but do not independently establish causal relationships.

Table 1. Operational definitions of structural-exposure motifs

| Motif | Matching unit | Required attributes | Structural matching criterion | Reference population | Threshold rationale |
| --- | --- | --- | --- | --- | --- |
| Generation-poor enrichment | Transmission branch and its endpoint substations | Total generator maximum capacity at each endpoint | The combined generation capacity at both endpoints is zero | All transmission branches; enrichment is referenced to the network-wide fraction satisfying this criterion | Identifies absence of endpoint generation support without introducing a fitted capacity cutoff. It does not measure generation adequacy relative to demand. |
| Low-capacity enrichment | Transmission branch | Branch rating, ce | 0<ce≤Q0.10c, where the quantile is calculated from positive branch ratings | Positive-rated branches determine the cutoff; all branches form the enrichment reference population | The lower 10% tail identifies relatively low-rated branches. The 5% and 20% tails are evaluated in sensitivity analysis. |
| Gas-coupled enrichment | Transmission branch and its endpoint substations | Mapped gas–power dependency links at each endpoint | At least one endpoint has a mapped gas-node connection in the gas–power coupling data | All transmission branches; enrichment is referenced to the network-wide fraction satisfying this criterion | A binary dependency criterion identifies branches incident to gas-linked substations without imposing a dependence-share cutoff. |
| High-reactance enrichment | Transmission branch | Absolute branch reactance, ¦xe¦ | ¦xe¦≥Q0.90(¦x¦) | All transmission branches determine both the cutoff and the enrichment baseline | The upper 10% tail identifies relatively high-reactance branches. The 5% and 20% tails are evaluated in sensitivity analysis. |
| Multi-constraint enrichment | Transmission branch | Six primitive structural-motif indicators | At least two of the following are satisfied: bridge, low redundancy, low capacity, high reactance, generation-poor, and gas-coupled | All transmission branches; enrichment is referenced to the network-wide fraction satisfying this combination rule | A fixed co-occurrence criterion identifies branches with multiple structural attributes. It does not imply that multiple operating constraints are simultaneously binding. |
| Low-redundancy enrichment | Transmission branch and its endpoint substations | Number of distinct incident branches at each endpoint, ku and kv | min(ku,kv)≤2 | All transmission branches; enrichment is referenced to the network-wide fraction satisfying this criterion | Uses limited endpoint connectivity as a topological proxy for low redundancy. Incident branches, including parallel circuits, are counted separately; degree does not directly measure alternative-path capacity. |
| Bridge-corridor enrichment | Transmission branch connecting distinct substations | Undirected substation connectivity and parallel-circuit multiplicity | The endpoint pair is a graph bridge, and exactly one branch connects that pair | The substation network determines bridge status; all transmission branches form the enrichment reference population | A connectivity-based criterion identifies branches whose removal disconnects their endpoints. Parallel circuits are excluded from bridge classification. |

Note: For each motif, qualifying branches are first identified from the static network without using terminal-severity labels. The initiating exposure set comprises directly affected branches and branches incident to affected or directly outaged substations. Substations mapped to affected or dependency-influenced gas nodes are also included. A structural-exposure motif is classified as enriched when at least one qualifying branch occurs in this set and its fraction exceeds the corresponding fraction across all network branches.

Propagation motifs

Propagation motifs are identified using deterministic rules applied to recorded branch-trip events. Their associations with terminal severity are then assessed using odds ratios stratified by disturbance configuration.

Table 2. Operational definitions of propagation motifs

| Propagation motif | Matching unit | Required attributes | Matching criterion | Threshold rationale |
| --- | --- | --- | --- | --- |
| Multi-round propagation | Cascade trajectory | Round indices of newly tripped branches | New branch trips occur in at least two distinct cascade rounds | Two failure-containing rounds distinguish continued propagation from trips confined to a single round. |
| Adjacent-round transition | Branch-trip sets in consecutive rounds | Round indices and endpoint-bus identifiers of tripped branches | At least one branch tripped in round r shares an endpoint bus with a branch tripped in round r+1 | Consecutive rounds and a shared endpoint identify temporal and spatial adjacency; they do not establish that one trip caused the other. |
| Same-round fan-out | Branch-trip set within one round | Branch identifiers, endpoint-bus identifiers, and trip rounds | At least three distinct branches tripped in the same round share an endpoint bus | Three branches distinguish a branching configuration from a two-branch connection. The criterion identifies coincident trips, not their causal order. |
| Parallel-corridor trip | Parallel branches within one round | Unordered endpoint-bus pairs and trip rounds | At least two branches with the same unordered endpoint-bus pair trip in the same round | Two branches are the minimum required to identify loss of multiple parallel circuits within one round. |
| Persistent-overload trip | Delayed branch-trip event | Trip mode and recorded overload-persistence counter | At least one delayed branch trip has a recorded overload-persistence count of at least two | The two-round threshold follows the simulator’s delayed-protection rule. Rounds represent successive solved states rather than elapsed physical time. |
| Mixed protection modes | Cascade trajectory | Trip-mode labels of branch-failure events | Both immediate and delayed branch trips occur in the trajectory | Identifies co-occurrence of the two modeled protection responses; no additional numerical threshold is imposed. |

Note: Each motif is recorded as a binary trajectory-level indicator and is present when its matching criterion is satisfied at least once. Unlike structural-exposure motifs, propagation motifs do not require a footprint-to-network enrichment comparison. Their associations with terminal severity are evaluated across all valid trajectories, stratified by disturbance configuration. The definitions reflect the modeled network and protection rules; applying them to another system requires reassessing the matching units, temporal resolution, and thresholds.